# Dynamic Design

February 9 Automotive Door Control System Design Khaled Ibrahim Abdulaziz

# 1.ECU1

## 1.1 State Machine for Components

## 1.2 State Machine for Operation

## 1.3 Sequence Diagram

## 1.4 CPU Load

Assume that execution time of each time = 1 ms

| Task Name | Periodicity (ms) | Execution Time (ms) | Occurrence |
| --- | --- | --- | --- |
| Door Task | 10 | 1 | 2 |
| Light Task | 20 | 1 | 1 |
| Speed Task | 5 | 1 | 4 |

Hyper period (H) = LCM = 20 ms

CPU Load = Sum(E) / H = (1 x 2 + 1 x 1 + 1 x 4) / 20 = 0.35

# 2.ECU2

## 2.1 State Machine for Components

2.2 State Machine for Operation

## 2.3 Sequence Diagram

## 2.4 CPU Load

Assume that execution time of each time = 1 ms

| Task Name | Periodicity (ms) | Execution Time (ms) | Occurrence |
| --- | --- | --- | --- |
| Control Task | 5 | 1 | 1 |

Hyper period (H) = LCM = 5 ms

CPU Load = Sum(E) / H = (1 x 1) / 5 = 0.2

# 3. BUS Load

CAN frame has about 125 bits, assuming we're using 500 kbit/s.

Time to transfer 1 bit Time = 1/bitrate = (1/500*1000) = 2 µs

Time to transfer 1 frame = 2 * 125 = 250 µs

1 frame / 20 ms -> 50 frames every 1000 ms, 1 frame / 10 ms -> 100 frames every 1000 ms, 1 frame / 5 ms -> 200 frames every 1000 ms

Total of frames per 1 second = 350 frames

Total time on Bus = total frames * time for frame = 350 * 0.25 = 87.5 ms

Bus Load = (87.5 / 1000) * 100= 8.75%